Quick response RTD temperature probes ( TM1102)

Applications：

The quick response RTD temperature probes are mainly used for occasions where require quick response. e.g. experimental facilities.

Technical data:

Connecting cable:  Teflon, temperature range -50℃~+260℃

Protection tube:    Stainless steel 316

Sensor element:    Pt100 meets IEC 751

Tolerance :        Class B: △t=±(0.30 + 0.005 |t| )℃

Class A: △t=±(0.15 + 0.002 |t| )℃

Response time：t0.9  approx. 2secs, in water 0.2 m/sec, diameter 1mm

接线图 Wiring diagram

单支2线 1x2-wire

单支3线 1x 3-wire

白色芯线 white core wire

红色芯线 red core wire

Dimensions(外形尺寸图）

Order Details:

| TM1102 | — | (1) | — | (2) | — | 000 | — | (4) | — | (5) | — | (6) | / | （7） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- (1) Protection tube diameter D

05      0.5mm

08      0.8mm

10      1.0mm

16      1.6mm

- (2) Insertion Length EL

100     100mm

200     200mm

Note：other lengths as per customers’ requitements( EL≤200mm)

- (4) Sensor element

11      1 x Pt100 in 2-wire circuit

13      1 x Pt100 in 3-wire circuit

- (5) Measuring range/connecting cable

14     -50℃～+260℃/Teflon

- (6) Cable length L    (L≥100mm）

100    100mm

200    200mm

2000   2000mm

Note：other lengths as per customers’ requirements

(100mm≤L<1000mm ,L1=50mm；L≥1000mm, L1=100mm)

- Additional options

00     No

01     strain relief spring/ strain relief spring hose

02     Shielded connecting cable(D≥4mm)

03     Waterproof function

07     Ferrules as per DIN 46 228 part4 (Standard)

08     Multipole connector (Providing types)

99     Special options

List the additional options in sequence, separated by commas.