Swaged RTD temperature probes  (TM1104)

Applications：

The swaged RTD temperature probes are used to measure the temperature of gases or liquids, including Heaters,Ovens, Mechanical equipments and Experimental facilities etc.

Technical data：

Connecting cable:  PVC, temperature range:-5℃~+85℃

Silicone, temperature range:-50℃~+180℃

Teflon, temperature range -50℃~+260℃

Glass Fiber, temperature range:-50℃~+350℃

Protection tube:    Stainless steel 304/316, OD=4mm, 5mm,6mm

Sensor element:    Pt100 meets IEC 751

Tolerance :        Class B: △t=±(0.30 + 0.005 |t| )℃

Class A: △t=±(0.15 + 0.002 |t| )℃

Response time：t0.9  approx. 4secs, in water 0.2 m/sec, diameter 4mm

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire 单支3线 1x 3-wire 双支4线 2x4-wire

单支4线 1x4-wire 双支6线 2x6-wire

白色芯线 white core wire 红色芯线 red core wire

Dimensions（外形尺寸图）

Order Details:

| TM1104 | — | (1) | — | (2) | — | 000 | — | (4) | — | (5) | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 4       4mm

5       5mm

6       6mm

- Insertion Length EL

- 100      100mm

200      200mm

500      500mm

Note：other lengths as per customers’ requitements( EL≤500mm)

- Sensor element

- 11       1 x Pt100 in 2-wire circuit

- 51       1 x Pt500 in 2-wire circuit

- 61       1 x Pt1000 in 2-wire circuit

- 13       1 x Pt100 in 3-wire circuit

- 14       1 x Pt100 in 4-wire circuit

- 12       2 x Pt100 in 4-wire circuit

- 16       2x Pt100 in 6-wire circuit

- Measuring range/Connecting cable

- 10      -5℃ ～ 85℃ /PVC

13      -50℃ ～ 180℃/Silicone

14      -50℃ ～ 260℃/Teflon

15      -50℃ ～ 350℃/Glass fiber

- Cable length L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

Note：other lengths as per customers’ requitements

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

- Additional options

- 00      No

01     Strain relief spring/strain relief spring hose

02     Shielded connection cable (D≥4mm)

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector (Providing types)

99      Special options

List the additional options in sequence, separated by commas.