Screw-in RTD temperature probes  (TM1105)

Applications：

The screw-in RTD temperature probes are used to measure the temperature of gases or liquids, including HVAC, Refrigeration equipments, Heaters, Ovens, Plants, Mechanical equipments and Experimental facilities,etc.

Technical data:

Connecting cable:  PVC, temperature range:-5℃~+85℃

Silicone, temperature range:-50℃~+180℃

Teflon, temperature range -50℃~+260℃

Glass Fiber, temperature range:-50℃~+350℃

Protection tube:    Stainless steel 304/316, OD=4mm,5mm, 6mm,8mm，9mm,11mm

Sensor element:    Pt100, Pt500, Pt1000 meets IEC 751

Tolerance :   Class B: △t=±(0.30 + 0.005 |t| )℃

Class A: △t=±(0.15 + 0.002 |t| )℃

Response time：t0.9  approx. 4secs, in water 0.2 m/sec, diameter 4mm

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire 单支3线 1x 3-wire 单支4线 1x4-wire

双支4线 2x4-wire 双支6线 2x 6-wire

白色芯线 white core wire 红色芯线 red core wire

Dimensions

Order Details:

| TM1105 | — | (1) | — | (2) | — | (3) | — | (4) | — | (5) | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(1)Protection tube diameter D

4       4mm

5       5mm

6       6mm

8       8mm

9       9mm

11      11mm

- Insertion Length EL

50      50mm

200     200mm

500     500mm

Note:other lengths as per customers’ requitements( 50mm≤EL≤500mm)

- Process connection  G

118      M12×1

119      M12×1.5

120      M14×1

121      M14×1.5

122      M16×1

123      M16×1.5

211      G1/4”

213      G1/2”

Details in “Thread code table”

- Sensor element

11      1 x Pt100 in 2-wire circuit

51      1 x Pt500 in 2-wire circuit

61      1 x Pt1000 in 2-wire circuit

13      1 x Pt100 in 3-wire circuit

14      1 x Pt100 in 4-wire circuit

12      2 x Pt100 in 4-wire circuit

16      2 x Pt100 in 6-wire circuit

- Measuring range/Connecting cable

10    -5℃ ～ +85℃ /PVC

13    -50℃ ～+ 180℃/Silicone

14    -50℃ ～ +260℃/Teflon

15    -50℃ ～ +350℃/Glass Fiber

- Cable length L（L≥100mm）

100     100mm

200     200mm

2000    2000mm

Note：other lengths as per customers’ requitements

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

- Additional options

00      No

01      Strain relief spring/strain relief spring hose

02      Shielded connecting cable (D≥4mm)

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector (Providing types)

99      Special options

List the additional options in sequence, separated by commas.