Slot RTD temperature probes(TM1109)

Applications：

The Slot RTD temperature probes are used to measure the temperature of the engine coil. It is mainly used to monitor coil temperature against damage.

Technical data:

Sensor element:    Pt100, Pt500, Pt1000 meets IEC 751

Tolerance :        Class B: △t=±(0.30 + 0.005 |t| )℃

Class A: △t=±(0.15 + 0.002 |t| )℃

Pressure range：2.5KV or 4KV（optional）

Operating voltage: ≤60V

Operating current：≤1mA

Connecting cable:PTFE(core),  Silicone(sheath)

Measuring range:-50℃~+180℃

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire单支3线 1x 3-wire

Dimensions(外形尺寸图)

Order Details:

| TM1109 | — | (1) | — | (2) | — | (3) | — | (4) | — | (5) | — | （6） | / | （7） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- （1）Fitting length L

50     50mm

100    100mm

- （2）Width W

9   9mm

- （3）Thickness d

3    3mm

- （4）Sensor element

11      1 x Pt100 in 2-wire circuit

13      1 x Pt100 in 3-wire circuit

- （5）Measuring range

13     -50 ℃~+180℃

- （6）Cable length L（L≥100mm）

100     100mm

200     200mm

2000    2000mm

Note：other lengths as per customers’ requirements.

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

- （7）Additional options

00      No

02      Shielded connecting cable (D≥4mm)

07      Ferrules as per DIN 46 228 part4 (Standard)

99      Special options

List the additional options in sequence, separated by commas.