Gasket RTD temperature probes（TM1110)

Applications：

The Gasket RTD temperature probes are used to measure the surface temperature of workpieces. It can be fixed with threaded.

Technical data

Connecting cable:  Silicone, temperature range:-50℃~+180℃

Teflon, temperature range -50℃~+260℃

Glass Fiber, temperature range:-50℃~+350℃

Protection tube:    copper

Sensor element:    Pt100, Pt500, Pt1000 meets IEC 751

Tolerance :        Class B: △t=±(0.30 + 0.005 |t| )℃

Class A: △t=±(0.15 + 0.002 |t| )℃

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire单支3线 1x 3-wire 单支4线 1x4-wire

双支4线 2x4-wire 双支6线 2x6-wire

白色芯线 white core wire 红色芯线 red core wire

Dimensions(外形尺寸图)

Order Details:

| TM1110 | — | 6 | — | 50 | — | 000 | — | （4） | — | （5） | — | （6） | / | （7） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Sensor element

- 11      1 x Pt100 in 2-wire circuit

51      1 x Pt500 in 2-wire circuit

- 61      1 x Pt1000 in 2-wire circuit

13      1 x Pt100 in 3-wire circuit

14      1 x Pt100 in 4-wire circuit

12      2 x Pt100 in 4-wire circuit

16      2 x Pt100 in 6-wire circuit

62      2 x Pt1000 in 4-wire circuit

- Measuring range/Connecting cable

13    -50℃ ～+ 180℃/Silicone

14    -50℃ ～ +260℃/Teflon

15    -50℃ ～ +350℃/Glass Fiber

（6）Cable length L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

Note：other lengths as per customers’ requirements.

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

- Additional options

- 00      No

01      Strain relief spring/strain relief spring hose

02      Shielded connecting cable (D≥4mm)

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector (Providing types)

99      Special options

List the additional options in sequence, separated by commas.