Shockproof RTD temperature probes TM1203

Applications:

Engines, Compressors, Refrigeration and shake devices.

Technical data

Connecting cable:  Silicone, temperature range:-50℃~+180℃

Protection tube:    Stainless steel 304/316

Sensor element:    Pt100, Pt500, Pt1000meets IEC 751

Tolerance :     Class B: △t=±(0.30 + 0.005 |t| )℃(standard)

Class A: △t=±(0.15 + 0.002 |t| )℃

Response time：t0.9  approx. 10secs, in water 0.2 m/sec, diameter 6mm

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire

白色芯线 white core wire

红色芯线 red core wire

Dimensions

Order Details:

| TM11203 | — | (1) | — | (2) | — | (3) | — | (4) | — | (5) | — | (6) | / | （7） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter  D

- 4mm

- 6     6mm

- Insertion Length EL

- 50    50mm

60  60mm

65    65mm

Note:other lengths as per customers’ requirements.

- Process connection G

- 118   M12 x 1

- 123   M16 x 1.5

- Details in “Thread code table”

- (4) Sensor element

11      1 x Pt100 in 2-wire circuit

51      1 x Pt500 in 2-wire circuit

61      1x Pt1000 in 2-wire circuit

(5)Measuring range/Connecting cable

13    -50℃ ～+ 180℃/Silicone

(6) Cable length L（L≥100mm）

100     100mm

200     200mm

2000    2000mm

Note:other lengths as per customers’ requirements.

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

(7)Additional options

00      No

07      Ferrules as per DIN 46 228 part4 (Standard)

99      Special options

List the additional options in sequence, separated by commas.