Form J Push-in RTD temperature probes with extension tube（TM1403)

Applications:

HVAC, Heating systems, Refrigeration and mechanical equipments etc.

Technical data

Protection tube：stainless steel 304/316

Sensor element:  Pt100, Pt500, Pt1000meets IEC 751

Tolerance :      Class B: △t=±(0.30 + 0.005 |t| )℃(standard)

Class A: △t=±(0.15 + 0.002 |t| )℃

Protection Class： IP54 or IP65

Response time：t0.9  approx. 14secs, in water 0.2 m/sec, diameter 6mm

接线图 Wiring diagram 铂电阻 RTD

单支2线 1x2-wire单支3线 1x 3-wire双支4线 2x4-wire

单支4线 1x4-wire安装螺钉 mounting screw 红色标记 red mark

Dimensions(外形尺寸图)

Order Details:

| TM1403 | — | (1) | — | (2) | — | (3) | — | (4) | — | (5) | — | (6) | / | （7） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- (1)protection tube diameter D and d

6       6mm

（d=6mm，D=8mm）

- （2）Insertion length EL

- 50     50mm

- 100    100mm

- 150      150mm

Note: other lengths as per customers’ requirements

- Process connection G

- 123   M16 x 1.5

- 213    G1/2”

- Details in “Threaded code table”

- (4) Sensor element

11     1 x Pt100 in 2-wire circuit

12     2 x Pt100 in 4-wire circuit

13     1 x Pt100 in 3-wire circuit

51     1 x Pt500 in 2-wire circuit

61     1 x Pt1000 in 2-wire circuit

62     2 x Pt1000 in 4-wire circuit

14     1 x Pt100 in 4-wire circuit

(5)Measuring range

20    -50℃ ～+ 400℃

(6) Cable length L（L≥100mm）

100     100mm

200     200mm

2000    2000mm

Note: other lengths as per customers’ requirements

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

(7)Additional options

00      No

02      shielded connecting cable

07      Ferrules as per DIN 46 228 part 4(standard)

08      Multipole connector (providing types)

99      Special options

List the additional options in sequence, separated by commas.