Applications：

analytical instruments， testing devices

Push-in Thermocouple  （TM2102)

Technical data:

Connecting cable:  Silicone, temperature range -50℃~+180℃

Glass fiber, temperature range -50℃~+350℃

Protection tube:    Stainless steel 304/316

Sensor element:   Thermocouple

接线图Wiring diagram

红 red 白 white

Dimensions（外形尺寸图）

Order Details:

| TM2102 | — | (1) | — | (2) | — | 000 | — | (4) | — | 00 | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 10       1.0mm

15       1.5mm

20       2.0mm

30       3.0mm

- Insertion length  EL

- 150      150mm

300      300mm

500      500mm

1000     1000mm

2000     2000mm

Note：other lengths as per customers’ requitements

- Sensor element

OE       Type E Thermocouple

- OK       Type K Thermocouple

- OJ        Type J Thermocouple

- OL        Type L Thermocouple

- OS        Type S Thermocouple

- OT        Type T Thermocouple

- OR        Type R Thermocouple

- Cable length  L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

(100≤L<1000 ,L1=50, L≥1000, L1=100)

Note：other lengths as per customers’ requitements

- Additional options

- 00      No

01      strain relief spring/strain relief spring hose

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector（providing types）

99      Special Options

List the additional options in sequence, separated by commas.