Screw-in thermocouple（TM2107)

Applications：

Heaters, Mechanical equipments of workshops, Plastic processing machineries.

Technical data

Connecting cable:  Shielded compensating cable

Protection tube:    Stainless steel 304

Sensor element:    Thermocouple

接线图-wiring diagram

红-red 白-white

Dimensions（外形尺寸图）

Order Details:

| TM2107 | — | (1) | — | (2) | — | （3） | — | (4) | — | 00 | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 6      6mm

8      8mm

10     10mm

- Insertion length  EL

25      25mm

EL ≦25mm

Note：other lengths as per customers’ requitements

- Process connection  G

111        M6

112        M8

115        M10×1

Details in “Thread code table”

- Sensor element

OE        Type E Thermocouple

- OK        Type K Thermocouple

- OJ         Type J Thermocouple

- OL        Type L Thermocouple

- OS        Type S Thermocouple

- OT        Type T Thermocouple

- OR        Type R Thermocouple

- Cable length  L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

(100≤L<1000 ,L1=50, L≥1000, L1=100)

Note：other lengths as per customers’ requitements

- Additional options

- 00      No

01      strain relief spring/strain relief spring hose

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector （providing type）

99      Special Options

List the additional options in sequence, separated by commas.