Form J thermocouple （TM2401)

Applications：

experimental facilities, industrial furnaces, ovens, heating devices.

Technical data:

Connecting cable:  Shielded compensating cable

Protection tube:    Stainless steel 304/316

Sensor element:    Thermocouple

Protection class :   IP54

接线图-wiring diagram

焊接-welding

红色标记- red mark   安装螺钉-mounting screws

Dimensions（外形尺寸图）

Order Details:

| TM2401 | — | (1) | — | (2) | — | 000 | — | (4) | — | (5) | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 2       2mm

- 3       3mm

4       4mm

5       5mm

6       6mm

Note：other specifications as per customers’ requitements

- Insertion length  EL

- 50     50mm

100    100mm

150    150mm

2000   2000mm

Note：other lengths as per customers’ requitements

- Sensor element

OE        Type E Thermocouple

- OK        Type K Thermocouple

- OJ         Type J Thermocouple

- OL        Type L Thermocouple

- OS        Type S Thermocouple

- OT        Type T Thermocouple

- OR        Type R Thermocouple

- ON        Type N Thermocouple

- Measuring range

- 21   -50 ～ +600℃

- Cable length  L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

Note：other lengths as per customers’ requitements

(100mm≤L<1000mm ,L1=50mm, L≥1000mm, L1=100mm)

- Additional options

- 00      No

01      strain relief spring/strain relief spring hose

03      Waterproof function

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector (providing type)

99      Special Options

List the additional options in sequence, separated by commas.