Form B/FormBUZ thermocouple（TM2501)

Applications：

Experimental facilities,Industrial furnaces, Ovens, Heating devices, Electric power and Metallurgy.etc.

Technical data:

Protection tube:    Stainless steel 304/316

Sensor element:   Thermocouple

Protection class :   IP65

Integrated temperature transmission(option) : 24VDC

Transmitter accuracy :±0.5%FS

接线图-wiring diagram

焊接-welding

红色标记- red mark   安装螺钉-mounting screws

Dimensions（外形尺寸图）

Order Details:

| TM2501 | — | (1) | — | (2) | — | 000 | — | (4) | — | (5) | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 2       2mm

- 3       3mm

4       4mm

6       6mm

8       8mm

Note：other specifications as per customers’ requitements

- Insertion length  EL

- 160     160mm

200     200mm

1000    1000mm

2000    2000mm

Note：other lengths as per customers’ requitements

- Sensor element

OE       Type E Thermocouple

- OK       Type K Thermocouple

- OJ        Type J Thermocouple

- OL        Type L Thermocouple

- OS        Type S Thermocouple

- OT        Type T Thermocouple

- OR        Type R Thermocouple

- ON        Type N Thermocouple

- Measuring range

- 21   -50 ～ +600℃

- 23    0 ～ +800℃

- 24    0 ～ +1000℃

- Cable length  L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

(100mm≤L<1000mm ,L1=50mm, L≥1000mm, L1=100mm)

Note：other lengths as per customers’ requitements

- Additional options

- 00      No

01      strain relief spring/strain relief spring hose

03      Waterproof function

06      Output 4~20mA in 2-wire

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector(providing type)

99      Special Options

List the additional options in sequence, separated by commas.