Thermocouple with extension tube （TM2503)

Applications：

Experimental facilities, Industrial furnaces, Heating boilers, Heating devices ,electric power and chemical industries.

Technical data:

Protection tube:    Stainless steel 304/316

Sensor element:   Thermocouple

Protection class :   IP65

Integrated temperature transmission(option) : 24VDC

Transmitter accuracy :±0.5%FS

接线图-wiring diagram

焊接-welding

红色标记- red mark   安装螺钉-mounting screws

Dimensions（外形尺寸图）

Order Details:

| TM2503 | — | (1) | — | (2) | — | (3) | — | (4) | — | (5) | — | （6） | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Protection tube diameter D

- 9      9mm

11      11mm

Note：other specifications as per customers’ requitements

- Insertion length  EL

- 160     160mm

200     200mm

500     500mm

1000    1000mm

2000    2000mm

Note：other lengths as per customers’ requitements

- Process connection G

- 133        M27 ×2

213        G1/2

150        M16×1.5-8(flexible cutting sleeve)

Details in “Thread code table”

- Sensor element

OE       Type E Thermocouple

- OK       Type K Thermocouple

- OJ        Type J Thermocouple

- Measuring range

- 21   -50 ～ +600℃

- 23    0 ～ +800℃

- Cable length  L（L≥100mm）

- 100     100mm

200     200mm

2000    2000mm

(100mm≤L<1000mm ,L1=50mm, L≥1000mm, L1=100mm)

Note：other lengths as per customers’ requitements

- Additional options

- 00      No

01      strain relief spring/strain relief spring hose

03      Waterproof function

06      Output 4~20mA in 2-wire

07      Ferrules as per DIN 46 228 part4 (Standard)

08      Multipole connector(providing type)

99      Special Options

List the additional options in sequence, separated by commas.